A/C service station cheap level, for gasoline cars

Model: ______________________ (cheap level)

Freon: R134A;

Main functions:

• Tank fill;

• Recovery;

• Vacuum;

• Oil injection manual;

• Charge;

• Manual air purge;

Specifications:

• Input power: 220 В ±10% -50/60 Гц;

• Compressor power: please indicate how many g/min (grams per minute).

And indicate the Model of compressor - this is important!);

• Recovery rate: >95%;

• Average gas state refrigerant recovery: 200 g/min;

• Vacuum pump capacity: 50 L/min;

• Gas cylinder capacity: 10 kg;

• Accuracy of gas cylinder load cel: ±10 g;

• New/Used oil bottle capacity: 400 ml;

• Drier-filter capacity: 600 ml;

• Max. Pressure: 20 bar;

• High pressure gauge range: -1 -30 bar;

• Low pressure gauge range: -1 -16 bar;

• Charge speed: 2 kg/min (max); ???

Features:

• Compressor made in China, power for agreement!

• Vacuum pump, made in China

• Charging hoses, normal quality China, length 3 meters;

• Heater band;